《脉冲电路与数字逻辑》实验报告

| 姓名 |  | 魏和琬 陈炫瑾 |  |  | 年级 | 2017 |
| --- | --- | --- | --- | --- | --- | --- |
| 学号 |  | 20174393 20174417 |  |  | 专业、班级 | 信安2班 物联1班 |
| 实验名称 | 七段数码管显示 |  |  |  |  |  |
| 实验时间 | 5.17 |  | 实验地点 | DS1410 |  |  |
| 实验成绩 |  |  | 实验性质 | □验证性 □设计性 □综合性 |  |  |
| 教师评价： □算法/实验过程正确； □源程序/实验内容提交 □程序结构/实验步骤合理； □实验结果正确； □语法、语义正确； □报告规范； 评语： 评价教师签名（电子签名）： |  |  |  |  |  |  |
| 一、实验目的 1、掌握 7 段数码管的显示原理，学会设计八进制数、十六进制数的七段数 码管显示； 2、学会使用显示模块和多路选择器设计组合逻辑电路。 |  |  |  |  |  |  |
| 二、实验项目内容 1、将拨码开关 SW3~SW0 以十六进制数显示在 1 个七段数码管上。 2、将拨码开关 SW11~SW9、SW8~SW6、SW5~SW3、SW2~SW0 分别以八进制数显示 在 4 个七段数码管上。 3、设计一个 3 位的 4 选 1 多路选择器，并利用拨码开关 SW15~SW14 控制选 择将拨码开关 SW11~SW9、SW8~SW6、SW5~SW3、SW2~SW0 以八进制数显示在 1 个七段数码管上。 |  |  |  |  |  |  |
| 三、实验过程或算法(包括真值表、原理图、关键步骤、核心代码） 共阳极七段数码管各个16进制数对应真值表 通过对应真值表，实现八、十六进制数显示模块,将拨码开关SW2~SW0、SW3~SW0 分别以八进制数、十六进制数显示在1 个七段数码管上。 实现一个 3 位的 4 选 1 多路选择器，并使所输出的值控制七段数码管，实现将 3 位的 4 选 1 多路选择器的 3 位输出接到八进制数显示模块上，最终实现利用拨码开关 SW15~SW14 控制选择将拨码开关SW11~SW9、SW8~SW6、SW5~SW3、SW2~SW0 以八进制数显示在 1 个七段数码管上。 代码： 十六进制数显示模块： module Five_1(sw0,sw1,sw2,sw3,segs,an); input sw0; input sw1; input sw2; input sw3; output reg [6:0] segs; output wire [7:0] an; wire[3:0] i; assign i[0]=sw0; assign i[1]=sw1; assign i[2]=sw2; assign i[3]=sw3; assign an[0]=0; always@(*) case(i) 4'h0:segs = 7'b000_0001; 4'h1:segs = 7'b100_1111; 4'h2:segs = 7'b001_0010; 4'h3:segs = 7'b000_0110; 4'h4:segs = 7'b100_1100; 4'h5:segs = 7'b010_0100; 4'h6:segs = 7'b010_0000; 4'h7:segs = 7'b000_1111; 4'h8:segs = 7'b000_0000; 4'h9:segs = 7'b000_1100; 4'ha:segs = 7'b000_1000; 4'hb:segs = 7'b110_0000; 4'hc:segs = 7'b111_0010; 4'hd:segs = 7'b100_0010; 4'he:segs = 7'b011_0000; 4'hf:segs = 7'b011_1000; default:segs = 7'b111_1111; endcase endmodule 八进制数显示模块： module Five_1(sw0,sw1,sw2,sw3,segs,an); input sw0; input sw1; input sw2; input sw3; output reg [6:0] segs; output wire [7:0] an; wire [2:0] oct; assign an[0]=0; assign oct[0]=sw0; assign oct[1]=sw1; assign oct[2]=sw2; always@(*) case(oct) 3'h0: segs = 7'b000_0001; 3'h1: segs = 7'b100_1111; 3'h2: segs = 7'b001_0010; 3'h3: segs = 7'b000_0110; 3'h4: segs = 7'b100_1100; 3'h5: segs = 7'b010_0100; 3'h6: segs = 7'b010_0000; 3'h7: segs = 7'b000_1111; default: segs = 7'b111_1111; endcase endmodule 3 位的 4 选 1 多路选择器的 3 位输出接到八进制数显示模块： 3 位的 4 选 1 多路选择器： module Change(oct0,oct1,oct2,oct3,ch,out); input [2:0] oct0,oct1,oct2,oct3; input [1:0] ch; output out; reg [0:2] out; always@(ch) begin case(ch) 2'b00:out=oct0; 2'b01:out=oct1; 2'b10:out=oct2; 2'b11:out=oct3; endcase end endmodule 八进制数显示模块： module Five_3(oct0,segs,an); input [2:0]oct0; output reg [6:0]segs; output [7:0] an; assign an[0]=0; assign an[1]=1; assign an[2]=1; assign an[3]=1; assign an[4]=1; assign an[5]=1; assign an[6]=1; assign an[7]=1; always@(oct0) begin case(oct0) 3'b000:segs = 7'b000_0001; 3'b001:segs = 7'b100_1111; 3'b010:segs = 7'b001_0010; 3'b011:segs = 7'b000_0110; 3'b100:segs = 7'b100_1100; 3'b101:segs = 7'b010_0100; 3'b110:segs = 7'b010_0000; 3'b111:segs = 7'b000_1111; default: segs = 7'b111_1111; endcase end endmodule 顶层模块： module Top(oct0,oct1,oct2,oct3,ch,segs,an); input [2:0] oct0,oct1,oct2,oct3; input [1:0] ch; output [6:0] segs; output [7:0] an; wire [2:0]out; Change ch1(oct0,oct1,oct2,oct3,ch,out); Five_3 show(out,segs,an); always@(*) begin end endmodule 管脚： set_property PACKAGE_PIN U13 [get_ports {an[7]}] set_property PACKAGE_PIN K2 [get_ports {an[6]}] set_property PACKAGE_PIN T14 [get_ports {an[5]}] set_property PACKAGE_PIN P14 [get_ports {an[4]}] set_property PACKAGE_PIN J14 [get_ports {an[3]}] set_property PACKAGE_PIN T9 [get_ports {an[2]}] set_property PACKAGE_PIN J18 [get_ports {an[1]}] set_property PACKAGE_PIN J17 [get_ports {an[0]}] set_property PACKAGE_PIN V10 [get_ports {ch[1]}] set_property PACKAGE_PIN U11 [get_ports {ch[0]}] set_property PACKAGE_PIN M13 [get_ports {oct0[2]}] set_property PACKAGE_PIN L16 [get_ports {oct0[1]}] set_property PACKAGE_PIN J15 [get_ports {oct0[0]}] set_property PACKAGE_PIN T18 [get_ports {oct1[2]}] set_property PACKAGE_PIN R17 [get_ports {oct1[1]}] set_property PACKAGE_PIN R15 [get_ports {oct1[0]}] set_property PACKAGE_PIN T8 [get_ports {oct2[2]}] set_property PACKAGE_PIN R13 [get_ports {oct2[1]}] set_property PACKAGE_PIN U18 [get_ports {oct2[0]}] set_property PACKAGE_PIN T13 [get_ports {oct3[2]}] set_property PACKAGE_PIN R16 [get_ports {oct3[1]}] set_property PACKAGE_PIN U8 [get_ports {oct3[0]}] set_property PACKAGE_PIN T10 [get_ports {segs[6]}] set_property PACKAGE_PIN R10 [get_ports {segs[5]}] set_property PACKAGE_PIN K16 [get_ports {segs[4]}] set_property PACKAGE_PIN K13 [get_ports {segs[3]}] set_property PACKAGE_PIN P15 [get_ports {segs[2]}] set_property PACKAGE_PIN T11 [get_ports {segs[1]}] set_property PACKAGE_PIN L18 [get_ports {segs[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[7]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {an[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {ch[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {ch[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct0[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct0[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct0[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct1[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct1[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct1[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct2[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct2[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct2[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct3[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct3[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {oct3[0]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[6]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[5]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[4]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[3]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[2]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[1]}] set_property IOSTANDARD LVCMOS33 [get_ports {segs[0]}] |  |  |  |  |  |  |
| 四、实验结果及分析和（或）源程序调试过程 拨码开关 SW3~SW0 以十六进制数显示在1 个七段数码管上。 拨码开关 SW2~SW0 以八进制数显示在 1 个七段数码管上 将 3 位的 4 选 1 多路选择器的 3 位输出接到八进制数显示模块上。 通过拨码开关sw14~15来选择拨码开关 SW11~SW9、SW8~SW6、SW5~SW3、SW2~SW0，从而显示出数字。 五、小组分工情况说明 魏和琬：内容讨论，代码和仿真部分完成。 陈炫瑾：内容讨论，状态转移图表绘制，脚管设置和上板。 |  |  |  |  |  |  |
| 六、个人总结 通过四选一选择器的原理来控制拨码开关，从而将八进制数显示在数码管上。运用顶层模块将两个选择器模块与显示模块结合。 |  |  |  |  |  |  |